# Appendix 1

## Basic Equipment and Drugs for Procedural Sedation and Analgesia in Children

All equipment should be checked regularly and stored in a cupboard.

### Devices to Administer Oxygen and Assist with Ventilation

| Item | Specification |
| --- | --- |
| Oxygen and oxygen tubing | Source must provide at least 90% oxygen via self-inflating positive pressure delivery system at 15 L/min for at least 60 minutes |
| Oxygen flow regulator |  |
| Nasal prongs | (+/- CO2 monitoring) |
| Venturi masks | To deliver 40% oxygen |
| Nebuliser and mask |  |
| Self-inflating resuscitation bag with reservoir for children |  |
| PEEP valve |  |
| Catheter mount |  |

### Airway Devices and Equipment

| Item | Specification |
| --- | --- |
| Face masks | Size 0, 1, 2, 3 and 4 |
| Laryngeal mask airways or similar supraglottic devices | Sizes 1.5–3 |
| Range of endotracheal tubes | Uncuffed (2.5–5.5 mm); Cuffed (4.0–7.0 mm) |
| Laryngoscope set | Two handles with adult and paediatric blades, spare batteries and bulbs |
| Water-soluble lubricant |  |
| 10 ml syringe for inflation of pilot balloon |  |
| Tape or equivalent to secure endotracheal tube |  |
| Oropharyngeal airways | Sizes 0–5 |
| Nasopharyngeal airways | Sizes 4 mm and 7 mm |
| Stylets/introducers/gum elastic bougie | Appropriately sized for endotracheal tubes |
| Magill forceps | Adult and paediatric size |

### Monitoring Equipment

| Item | Specification |
| --- | --- |
| ECG monitor and cardiac defibrillator | With conductive paste or pads, paddles and electrodes |
| Pulse oximeter | Adult and paediatric probes |
| Blood pressure monitoring device | Non-invasive, with appropriately sized paediatric and adult cuffs |
| Stethoscope / precordial stethoscope |  |
| Thermometer | Including low-reading capability |
| Blood glucose testing device |  |
| Selection of test tubes | For blood biochemistry and full blood count |
| Capnograph | Nasal prongs with capnography line strongly recommended, but not compulsory |

### Equipment with which to Gain Intravenous Access

| Item | Specification |
| --- | --- |
| Gloves |  |
| Tourniquet |  |
| Sterile gauze pads |  |
| Alcohol skin wipes |  |
| Intravenous cannulae | 24–18 G |
| Sterile needles |  |
| Assortment of syringes | 1–50 ml |
| Sharps container |  |
| Paediatric intraosseous needles |  |
| Tape or equivalent to secure IV cannulae |  |

### Equipment for Accurate Infusion of Drugs and Fluids

| Item | Specification |
| --- | --- |
| Infusion pumps | Intravenous fluid and drug administration |
| Syringe drivers | Drug administration in advanced sedation |
| Intravenous administration sets | Paediatric administration sets and buretrol |
| Stickers, marker pen for labelling syringes |  |
| Drip stands |  |
| Intravenous fluids | Crystalloids and colloids |

### Hardware and Miscellaneous Equipment

| Item | Specification |
| --- | --- |
| Source of suction | Including connection tubing |
| Suction catheters | Including catheters for suctioning ET tubes, and Yankauer-type suction nozzles |
| Therapeutic heat source | Desirable for long cases |
| Cardiac arrest board |  |
| Appropriate lighting | Adequate for intravenous access |
| Operating surface that can be tilted | Capable of Fowler’s and Trendelenburg positions |
| Urinary catheters |  |
| Nasogastric tubes |  |
| Medication stickers |  |
| Means of summoning emergency assistance |  |
| SA Resuscitation Council algorithms | BLS, ALS for adults and children, anaphylaxis management and choking |
| Resuscitation documentation record |  |

### Recommended Emergency Drugs

Drug

Naloxone

Flumazenil

Adrenaline (at least 10 ampoules)

Atropine or glycopyrrolate

Ephedrine or phenylephrine (or other alpha agonist)

Lignocaine

Glucose 50%

Hydrocortisone, methylprednisolone or dexamethasone

Promethazine (or equivalent H1-antagonist)

Salbutamol

Suxamethonium

Intralipid

Calcium-channel blocker (e.g. nifedipine)

Beta blocker (e.g. esmolol)

Selective alpha-1 adrenergic and non-selective beta-adrenergic receptor blocker (e.g. labetalol)

Specific antagonists must be immediately available where an SP administers opioid analgesics and/or benzodiazepines for PSA, regardless of the route of administration. After reversal, patients must be monitored for a longer period in the recovery room to prevent re-sedation.